# Advance Noga Sensor

# Software Requirements Specification (SRS)

AOP-52 Compliant Template

# Document Control

| Document Title | Software Requirements Specification |
| --- | --- |
| Project Name | Advance Noga Sensor Software |
| Document ID | SRS |
| Version | 0.1 |
| Date | 28/01/2026 |
| Classification | Unclasified |
| Author(s) | Erez Shabtai |
| Approved By | Dmitri Poverenov[Name] |
| Organization | Orion Advance System[Organization Name] |

# Document History

| Version | Date | Author | Changes | Approval |
| --- | --- | --- | --- | --- |
| 0.1 | 28/01/2026 | Erez Shabtai | Initial draft | - |
| 0.2 | 22/02/2026 | Erez Shabtai | After requirements review | - |
| 0.3 | 09/03/2026 | Erez Shabtai | After additional comments received by mail | - |

Contents

Document Control	2

Document History	2

1. Introduction	5

1.1 Purpose	5

1.2 Scope	5

1.3 Definitions, Acronyms, and Abbreviations	6

1.4 References	6

1.5 Overview	6

2. Overall Description	7

2.1 Product Perspective	7

2.2 Product Functions	8

2.3 User Characteristics	8

2.4 Constraints	8

2.5 Assumptions and Dependencies	8

2.6 Safety and Security Requirements	9

3. Specific Requirements	10

3.1 Functional Requirements	10

3.1.1 Bus Drivers	10

3.1.2 Device Drivers	14

3.1.3 Algorithm Implementation	15

3.1.4 Communication protocol requirements	16

3.1.5 Telemetry protocol requirements	16

3.1.6 Init module requirements (including initiate BIT)	17

3.1.7 application logic requirements	20

3.1.8 Test mode	21

3.1.9 CLI	22

3.2 Performance Requirements	22

3.3 Interface Requirements	23

3.4 Safety Requirements	23

3.5 Security Requirements	24

3.6 Software Quality Attributes	24

4. Verification and Validation	24

4.1 Verification Methods	24

4.2 Validation Criteria	24

4.3 Traceability Matrix	24

5. Supporting Information	25

5.1 Appendices	25

Appendix A: Requirements Template	25

Appendix B: Requirement Categories	25

Appendix C: AOP-52 Compliance Checklist	25

Document Approval	26

# 1. Introduction

## 1.1 Purpose

This Software Requirements Specification (SRS) document provides a complete description of all software requirements for the Fuze Advance Noga Sensor system. This document is compliant with NATO Allied Ordnance Publication AOP-52 "Software Safety Assessment of Defense Systems Containing Programmable Electronics."

Intended Audience:

- Software developers

- System engineers

- Safety engineers

- Quality assurance personnel

- Project managers

- Certification authorities

- End users

## 1.2 Scope

Software Product Name: Advance Noga Sensor

Product Description:

The sensor has an electrical interface with the weapon system, which is carried out through a 13-pin connector located on the sensor. Through this connector, the weapon system provides operating voltage to the sensor, sends commands to the sensor, and receives responses from it. Communication is based on the master/slave principle, with the weapon system serving as the master and the sensor as the slave. The communication channel is a two-way serial asynchronous communication channel according to the RS-422 standard.

The sensor has an additional electrical interface, facing the fuze installed in the system, which allows the sensor to directly provide an activation signal to the fuze with the required height identification. This interface uses the same 13-pin connector, with the system node routing the activation signal to the fuze connector.

Safety Criticality:

This software is classified as SIL-3 (IEC 61508) and SSCI-2 (AOP-52).

## 1.3 References

Applicable Documents:

|  | Document/standard name | Description |  |
| --- | --- | --- | --- |
| 1 | NATO AOP-52, "Software Safety Assessment of Defence Systems Containing Programmable Electronics" |  |  |
| 2 | IEC 61508, "Functional Safety of Electrical/Electronic/Programmable Electronic Safety-related Systems" |  |  |
| 3 | N ממשק חשמלי ותקשורת למחוש קרבה עבור מערכת (D11).docx | System requirements document |  |
| 4 | 84r023601.pdf | HW schematics |  |

## 1.4 Overview

This SRS is organized according to AOP-52 requirements for safety-critical software documentation. All requirements are uniquely identified using the format: REQ-[Category]-[Number] (e.g., REQ-FUNC-001)

# 2. Overall Description

## 2.1 Product Perspective

## 2.2 Product Functions

The software shall provide the following major functions:

- Initiate the Advance Noga sensor

  - Initiate all peripherals

  - Initiate algorithm

  - Built In Test at start

- Implement algorithm to constantly measure height from analog samples

- Save analog samples in real time to nonvolatile flash memory

- Constant Built In Test – TBD if required

- Communicate with the WP through UART (RS422)

- Decide when and activate fuze

- Test mode

- Firmware update

## 2.3 User Characteristics

## N/A

## 2.4 Constraints

Regulatory Constraints:

Compliance with AOP-52 software safety requirements

Compliance with MISRA

Develop bare metal (no RTOS)

Export control regulations: ITAR, EAR

Hardware Limitations:

Processor: STM32H7A3 280MHz

Memory: STM32H7A3 internal ram + internal flash, 0.5MB FRAM (CY15B104QSN)

Power: Maximum 150 mA

Vcc = 3.3v

Record samples in real time

## 2.5 Assumptions and Dependencies

Assumptions:

TBD

Dependencies:

- TBD

## 2.6 Safety and Security Requirements

Safety Integrity Level: This software is classified as SIL-3 (IEC 61508) and SSCI-2 (AOP-52)

Safety Mechanisms:

Watchdog timer monitoring

Built-in self-test (BIST)

# 3. Specific Requirements

## 3.1 Functional Requirements

### 3.1.1 Bus Drivers

This subsection specifies the serial bus driver functional requirements for all communication interfaces on the STM32H7A3 microcontroller. Each driver shall be initialized, configured, and monitored in accordance with the interface-specific parameters defined below.

#### 3.1.1.1 UART Requirements

The following requirements cover the initialization, configuration, baud-rate accuracy, and error handling for all UART/USART peripherals used by the Orion Advance Noga Sensor system.

REQ-FUNC-001: USART1 RS-422

Input:

Data received from WP through RS-422 UART input port.

Communication responses to transmit to the RS-422 UART output port

Process:

The SW shall initialize USART1 in asynchronous mode configured for RS-422 full-duplex communication with the WS module. The peripheral shall be initialized with the parameters defined in Table 3-1 upon firmware startup, prior to any bus transaction.

Table 3-1: USART1 RS-422 Configuration

| Parameter | Value |
| --- | --- |
| Peripheral | USART1 (APB2) |
| Mode | Asynchronous |
| Baud Rate | 115200 baud |
| Data Bits | 8 |
| Parity | odd |
| Stop Bits | 1 |
| Flow Control | None (RS-422 framing) |
| Duplex | Full-duplex (RS-422) |
| DMA | TX and RX support (using will be decided in design) |

Acceptance Criteria:

USART1 peripheral registers match the configuration in Table 3-1 after initialization. A loopback test frame is transmitted and received.

Output:

Functions returns OK/fail with failure reason

UART Read: Request to communication module parser

UART Write: Response sent to WS through RS-422 UART output port.

If function failed, alert HW error through status message and add debug alert message. This is applicable to all fail driver functions

REQ-FUNC-002: UART5 Debug Port @ Test Connector

Input:

CLI requests from serial terminal

Process:

The SW shall initialize UART5 as a CLI debug port with the parameters defined in Table 3-2.

Table 3-2: UART5 Debug Port Configuration

| Parameter | Value |
| --- | --- |
| Peripheral | UART5 (APB1) |
| Mode | Asynchronous |
| Baud Rate | 115200 baud |
| Data Bits | 8 |
| Parity | None |
| Stop Bits | 1 |
| Direction | RX and TX |
| Production | Output disabled |
| DMA | TX enabled |

Acceptance Criteria:

Response to any CLI request.

Output:

CLI response to serial terminal

Debug prints to serial terminal

REQ-FUNC-003: Baud Rate Accuracy

Input:

N/A

Process:

The baud rate generated by each UART/USART peripheral shall be accurate to within +/- 2 % of the nominal value specified in its respective configuration table, across the full operating temperature range of the STM32H7A3 (0°C to +85°C). The accuracy calculation shall account for the APB clock source frequency and the BRR register value.

Acceptance Criteria:

Measured baud rate on each UART TX line lies within the +/- 2 % tolerance band at 0°C, +25°C, and +85°C ambient temperatures, verified with a calibrated oscilloscope or protocol analyzer.

Output:

N/A

REQ-FUNC-004: UART Error Detection and Handling

Input:

N/A

Process:

The UART driver shall detect and report the error conditions listed in Table 3-3. Upon detection, the driver shall assert the corresponding error flag, log the event via the diagnostic subsystem, and execute the specified recovery action without loss of subsequent valid frames.

Table 3-3: UART Error Conditions and Recovery

| Error Condition | Flag | Recovery Action |
| --- | --- | --- |
| Framing Error | USART_SR_FE | Discard frame; resume receive |
| Overrun Error | USART_SR_ORE | Flush RX FIFO; assert alarm |
| Parity Error | USART_SR_PE | Discard frame; request retransmit |
| Timeout (RX) | USART_TIMEOUT | Reset RX state; log timeout |

Acceptance Criteria:

Output:

N/A

#### 3.1.1.2 GPIO Requirements

REQ-FUNC-005: GPIO

Input:

N/A

Process:

The SW will use STM32 HAL library GPIO driver as is. Any configuration will be described in the device driver (that will use the GPIO bus driver) requirements

Output:

N/A

#### 3.1.1.3 SPI Requirements

REQ-FUNC-006: SPI

Input:

N/A

Process:

The SW will use STM32 HAL SPI driver as is. Any configuration will be described in the device drivers (that will use the SPI bus driver) requirements

Output:

N/A

#### 3.1.1.4 ADC Requirements

REQ-FUNC-007: ADC

Input:

N/A

Process:

The SW will use STM32 HAL ADC driver as is. Any configuration will be described in the device driver (that will use the ADC bus driver) requirements

Output:

N/A

### 3.1.2 Device Drivers

#### 3.1.2.1 QSPI FRAM Flash driver CY15B104QSN

Device driver requirements for Infineon’s CY15B104QSN QSPI FRAM (nonvolatile memory)

REQ-FUNC-008: FRAM init

Input:

N/A

Process:

The SW shall initialize CY15B104QSN FRAM through QSPI bus and configure it to use DMA for read and write to the device.

Acceptance Criteria:

Get correct device id from the FRAM.

Output:

Functions return OK/fail with failure reason

REQ-FUNC-009: FRAM read

Input:

Empty data buffer to read FRAM data to.

Process:

The SW shall read data buffer from the FRAM. Using DMA is not mandatory – there is no performance requirement.

Acceptance Criteria:

Read from FRAM known buffer and compare.

Output:

Functions return OK/fail with failure reason

REQ-FUNC-010: FRAM write

Input:

Data buffer to write to the FRAM

Process:

The SW shall write data buffer to the FRAM using DMA as efficiently as possible (performance requirements is detailed).

Acceptance Criteria:

Read from FRAM the write data and make sure it changes.

Output:

Functions return OK/fail with failure reason

#### 3.1.2.2 PLL init/configure

Device driver requirements for PLL configuration using SPI BUS

REQ-FUNC-011: PLL configure

Input:

Frequency to initiate the PLL with

Process:

The SW shall initialize the PLL with its entire configuration (chirp and all its related parameters) using SPI bus.

Acceptance Criteria:

Correct PLL frequency measured by scope/test equipment

Output:

Functions return OK/fail with failure reason,

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

### 3.1.3 Algorithm Implementation

REQ-FUNC-012: [height detection algorithm implementation]

Description:

Implement the height detection algorithm received from the algorithm developer.

The SW programmer shall get from the algorithm developer:

Full implementation of the algorithm in c code. With clear definition of the algorithm API

Full example how to run the code on PC

Test vectors as input to run the algorithm on.

Expected output vectors.

A clear pass/fail definition (should not require bit-exact matching due to floating point implementation and differences in FFT implementation.)

Input:

The height / Velocity / Delay requirement detection module shall get as input:

Test vectors as input to run the algorithm on.

Process:

The SW shall implement the C code algorithm, run the algorithm on the input test vectors and produce output vectors/results.

Output:

The SW output vector should be close to the test output vectors according to the defined algorithm pass definition (from the algorithm developer).

Note: The Purpose of this requirement is to check that the algorithm implementation code on the MCU is close enough to the PC reference that received from the algorithm developer. Please note that the input for this requirement is the test vector and not the analog input samples (for that we have separate requirement in the document). Therefor it is expected to get the required output of the algorithm and the pass criteria (what to compare to –temp results in some points inside the algorithm, distance, time, decision if to pull trigger, etc)  from the algorithm developer

### 3.1.4 Communication protocol requirements

REQ-FUNC-013: RS-422 Communication Protocol

Description:

The SW shall implement the communication protocol as described in N ממשק חשמלי ותקשורת למחוש קרבה עבור מערכת (D11).docx document.

The SW shall function as slave only, respond to WS requests through the RS422 UART interface, and will not initiate messages by itself.

Input:

The SW Shall receive request from the WS through RS422 UART interface.

Process:

The SW shall decode the UART message, call the required operation and send the corresponding response to the WS through the RS422 UART interface.

Output:

Correct corresponding UART response for each request according to the protocol.

### 3.1.5 Telemetry protocol requirements

REQ-FUNC-014: Telemetry Communication protocol

Description:

The SW shall implement the telemetry communication protocol as described in N ממשק חשמלי ותקשורת למחוש קרבה עבור מערכת (D11).docx document.

Input:

The SW Shall receive request from the WS through RS422 UART interface.

Process:

The SW shall function as SPI master, sending the telemetry data to the telemetry system through the SPI interface.

The SW shall get the telemetry data according to N ממשק חשמלי ותקשורת למחוש קרבה עבור מערכת (D11).docx document.

SPI clock is 1MHz.

Period between 4 bytes messages is according to modulating signal end of gradient (rise or fall), but not less than 180 usec (as described in the קרבה עבור מערכת (D11).docx document).

Output:

SPI message to telemetry system.

REQ-FUNC-015: Telemetry PROX IBIT test signal

Description:

The SW shall implement the telemetry test signal as described in N ממשק חשמלי ותקשורת למחוש קרבה עבור מערכת (D11).docx document.

Input:

The SW Shall receive PROX IBIT request from the WS through RS422 UART interface.

Process:

The SW shall function as SPI master, sending the test data to the telemetry system through the SPI interface.

The SW shall get the test data according to N ממשק חשמלי ותקשורת למחוש קרבה עבור מערכת (D11).docx document.

SPI clock is 1MHz.

Period between 4 bytes messages is according to PROX IBIT command.

Output:

SPI message to telemetry system.

### 3.1.6 Init module requirements (including initiate BIT)

REQ-FUNC-016: init sequence

Description:

Input:

N/A

Process:

The SW shall perform the init sequence:

- FRAM Flash init

- PLL init

- Analog voltage input validation (pass/ fail only)

- BIT tests

- RS422 communication init

- Telemetry communication init

- CLI init

Output:

Return OK/Fail with fail reason

REQ-FUNC-017: RAM BIT

Description:

Input:

N/A

Process:

The SW shall perform “walking 1s and 0s” test to internal MCU RAM

Output:

Return OK/Fail with fail reason

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

REQ-FUNC-018: Internal flash BIT

Description:

Input:

N/A

Process:

The SW shall perform “walking 1s and 0s” test to internal MCU Flash

Output:

Return OK/Fail with fail reason

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

REQ-FUNC-019: External flash BIT

Description:

Input:

N/A

Process:

The SW shall perform “walking 1s and 0s” test to external FRAM Flash

Output:

Return OK/Fail with fail reason

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

REQ-FUNC-020: PLL BIT

Description:

Input:

N/A

Process:

The SW shall perform PLL test (TBD - to be defined by Orion before design review)

Output:

Return OK/Fail with fail reason

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

REQ-FUNC-021: 5V RX test

Description:

Check that 5V RX exist in digital input

Input:

N/A

Process:

The SW shall read the 5V RX digital input and verify it is set high.

Output:

Return OK/Fail with fail reason

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

REQ-FUNC-022: 5V TX test

Description:

Check that 5V TX exist in digital input

Input:

N/A

Process:

The SW shall read the 5V TX digital input and verify it is set high

Output:

Return OK/Fail with fail reason

If failed, the SW shall not start the program except the communication port. And set status for fatal error. Also, add corresponding debug message.

### 3.1.7 application logic requirements

REQ-FUNC-023: Receive Analog Data

Description:

Continuously read analog sensor input using DMA. Target is to receive and handle 5888 samples every 3 milliseconds.

Note: 3 milliseconds is the preferred required rate. If not possible, the maximal period is 3.6 milliseconds

Input:

Data from analog sensor input pin

Process:

The SW shall continuously read through DMA data from the analog sensor pin.

Output:

5888 samples buffers in RAM to send to the algorithm and save to flash

REQ-FUNC-024: Run the algorithm on received data

Description:

Run the height detection algorithm on the received analog sensor input.

Input:

Buffers of 5888 samples of analog sensor input

Process:

The SW shall implement the C code algorithm (received from the algorithm developer), run the algorithm on the input analog vectors and produce output vectors/results.

Output:

Calculated height,

Decision if to activate the fuze

REQ-FUNC-025: ADC Samples logging

Description:

Continuously save analog sensor input to FRAM flash using DMA

Input:

Buffers of 5888 samples of analog sensor input

Process:

Upon reaching a specific configured height, the SW shall save all analog sensor input samples to the FRAM flash memory in real time using DMA.

Output:

Write to flash the analog sensor samples

REQ-FUNC-026: Fuze activate decision

Description:

Input:

Height,

Activate threshold

Process:

Once the height is within activate threshold range, the SW shall activate the Fuze using dedicated GPIO (HOB_Det_uC)

Output:

Activate GPIO and HOB_Det_FUZE

INFO: Continuous BIT – Not required in first phase, will be defined in later phase

INFO: PLL control during main loop – Not required in first phase, will be defined in later phase

### 3.1.8 Test mode

REQ-FUNC-027: Test mode

Description:

The SW shall support test mode through the RS422 interface. The exact test mode requirements will be defined later in the project

Input:

TBD

Process:

TBD

Output:

TBD

### 3.1.9 CLI

REQ-FUNC-028: CLI

Description:

The SW shall support CLI using the debug UART. The CLI task shall wait for requests from external serial terminal (putty, Tera Term, python simulator, etc) and respond to them.

CLI requests shall be defined later in the project and shall include:

- Bring up commands to test the HW

- Configure parameters on the board

- Run unit tests on the board

- Firmware update from WS (RS-422)

In addition, all debug prints shall be output during runtime in debug mode through the CLI without request

Input:

TBD

Process:

TBD

Output:

TBD

## 3.2 Performance Requirements

REQ-PERF-001: Processing Time

The system shall complete the main loop cycle within 3 milliseconds total, including:

- Process input data of 5888 ADC samples using the algorithm

- Produce output height

- Write the samples data to FRAM

- Send telemetry data through SPI at 180 usec intervals

Acceptance Criteria:

Measured latency < 3 ms in 99.9% of test cases

No missed deadlines under maximum load

## 3.3 Interface Requirements

REQ-IF-HW-001: Hardware

The SW shall run on Orion Sensor HW board, on STM32H7A3 MCU

REQ-IF-HW-002: HOST

The SW shall communicate with the WS host using RS422 UART interface

REQ-IF-HW-003: Fuze activation

The SW shall activate the Fuze using HOB_Det_uC GPIO

REQ-IF-HW-004: Telemetry system

The SW shall communicate with the telemetry system using telemetry SPI bus.

REQ-IF-HW-005: CLI

The SW shall communicate with the CLI using the debug UART port

## 3.4 Safety Requirements

Safety requirements are critical to AOP-52 compliance and must be rigorously verified.

REQ-SAFE-001: Watchdog Monitoring

The software shall service the hardware watchdog timer at least once every 10 milliseconds. Failure to service the watchdog within this period shall result in a hardware-initiated system reset.

Failure Mode:

- Failure: Watchdog not serviced within timeout period

- Effect: System reset (safe state)

- Mitigation: Independent watchdog timer in hardware

REQ-SAFE-002: MISRA

The software shall comply with MISRA C:2012 coding standards for all code in the project

REQ-SAFE-003: Static analysis

The software shall undergo static analysis for all code in the project

REQ-SAFE-004: Code review

All software code shall undergo code review by Tandem/Orion before integration.

Note – all bugs discovered in code review/testing before each formal release will be summarized in the release test report

## 3.5 Security Requirements

N/A

## 3.6 Software Quality Attributes

- Reliability

- Maintainability

- Portability

- Testability

- Unit testing - to be defined in test plan. The guideline is to > test all units in the SW

- Code coverage - guideline is to test as much as possible

# 4. Verification and Validation

## 4.1 Verification Methods

Per AOP-52, each requirement shall be verified using one or more of the following methods:

| Method | Code | Description |
| --- | --- | --- |
| Test | T | Execute software under controlled conditions |
| Inspection | I | Visual examination of code/documentation |
| Analysis | A | Mathematical or logical analysis |
| Demonstration | D | Operational demonstration |

## 4.2 Validation Criteria

Validation confirms software meets operational needs and system requirements.

## 4.3 Traceability Matrix

The Requirements Traceability Matrix (RTM) ensures bidirectional traceability between:

Software requirements → Design elements – will be filled in design document

Software requirements → Testing document – will be filled in testing tocument

# 5. Supporting Information

## 5.1 Definitions, Acronyms, and Abbreviations

| Term/Acronym | Definition |
| --- | --- |
| ADC | Analog-to-Digital Converter |
| API | Application Programming Interface |
| AOP-52 | Allied Ordnance Publication 52 - Software Safety Assessment |
| BIT | Built-In Test |
| CLI | Command Line Interface |
| DMA | Direct Memory Access |
| FFT | Fast Fourier Transform |
| FMEA | Failure Mode and Effects Analysis |
| FRAM | Ferroelectric Random Access Memory |
| FTA | Fault Tree Analysis |
| GPIO | General Purpose Input/Output |
| HAL | Hardware Abstraction Layer |
| MISRA | Motor Industry Software Reliability Association (C/C++ coding standards) |
| MCU | Microcontroller Unit |
| PLL | Phase-Locked Loop |
| QSPI | Quad Serial Peripheral Interface |
| RTOS | Real-Time Operating System |
| SIL | Safety Integrity Level |
| SPI | Serial Peripheral Interface |
| SSCI | Software Safety Criticality Index |
| SRS | Software Requirements Specification |
| UART | Universal Asynchronous Receiver/Transmitter |
| USART | Universal Synchronous/Asynchronous Receiver/Transmitter |
| V&V | Verification and Validation |

## 5.2 Appendices

### Appendix A: Requirements Template

Use this template for documenting new requirements.

### Appendix B: Requirement Categories

| Prefix | Category | Description |
| --- | --- | --- |
| FUNC | Functional | What the software must do |
| PERF | Performance | Speed, throughput, latency |
| IF-HW | Hardware Interface | Hardware connections |
| IF-SW | Software Interface | Software APIs |
| SAFE | Safety | Safety-critical requirements |
| SEC | Security | Authentication, encryption |
| QUAL | Quality | Reliability, maintainability |

### Appendix C: AOP-52 Compliance Checklist

☑ Safety integrity level defined

☑ Safety requirements identified

☑ Verification methods specified

Traceability matrix not needed at requirements

☑ All requirements uniquely identified

☑ Requirements written as testable "shall" statements

# Document Approval

Prepared By:

| Name | Role | Signature | Date |
| --- | --- | --- | --- |
| [Name] | Requirements Engineer |  |  |

Reviewed By:

| Name | Role | Signature | Date |
| --- | --- | --- | --- |
| [Name] | Safety Engineer |  |  |
| [Name] | System Engineer |  |  |
| [Name] | Software Lead |  |  |

Approved By:

| Name | Role | Signature | Date |
| --- | --- | --- | --- |
| [Name] | Project Manager |  |  |
| [Name] | Quality Manager |  |  |

This template complies with:

• NATO AOP-52: Software Safety Assessment

• IEC 61508: Functional Safety

• IEEE 830-1998: Software Requirements Specifications